Drydock Providence

Description

The drydock construction was completed in late 2021. It is 500 ton constructed of ASTM A-36 steel with 4 pairs of port and starboard ballast tanks. Framing is transverse in the pontoon and wing wall, with longitudinal framing on the safety deck and top deck.  It incorporates a shore power connection, onboard generator, individual pumps, and one end of the drydock is raked to improve towing. Each tank has a flood valve operated from the top deck and is deballasted using a submersible pump in each tank. Crossover valves are located between tanks to allow emergency pumping from an adjacent tank. The dry dock has a Libra GRP Fiberglass Bridge Wing Console on the starboard wing. The control console contains all pump start/stop controls and draft reading monitors. Pump starters and electrical panels are in the safety deck space. A generator is located on the starboard side top deck to provide power for dry docking. Shore power will be supplied to the dock when moored, and shore power for the docked vessel will be provided through the dry dock distribution system.  The top deck is equipped to operate the dry dock and position the docked vessel.  Four winches are located on each side with valve handwheels to operate the ballasting valves. The safety deck is not an occupied space but does contain power panels and motor starters.